# ANALOG & DIGITAL LINE ISOLATOR

# MODEL AD-4865

## PRODUCT DESCRIPTION AND INSTALLATION DETAILS

# ACA PERMIT

Design Two Thousand Pty Ltd ACA Supplier’s Code	N468

# DESCRIPTION

The Analog & Digital Line Isolator AD-4865 is a combination of the Design Two Thousand LI-4796 and the DI-4840R Line Isolation cards supplied in a 1RU 19” sub-rack.

The Analog & Digital Line Isolator AD-4865 is used for isolating audio equipment connected to 2 or 4 wire leased telephone lines, providing dc isolation and ac audio paths, and isolating data equipment connected to Dedicated Data Service (DDS) lines, providing isolation for RS232 appliances and also eliminating ground loop problems.

This unit provides two analog 4 wire circuits and one digital asynchronous RS232 circuit in a 1 unit high 19" sub-rack. The inputs (equipment) and outputs (line) are provided on RJ45 connectors.

The AD-4865 guards against dangerous voltages being passed from Customer Equipment to the Line. The AD-4865 also reduces the risk of electric shock to the customer and/or damages to Customer Equipment, should dangerous voltages be accidentally imposed on the line side.

The AD-4865 provides greater than 3500 Volts AC and greater than 5000 Volts DC isolation for more than one minute between the Equipment and Line.

# CONNECTION

4W ANALOG

| Equipment (SK1) | Tx | Pair 1 | Pins 4 & 5 |
| --- | --- | --- | --- |
|  | Rx | Pair 2 | Pins 3 & 6 |
| Line (SK2) | Tx | Pair 1 | Pins 4 & 5 |
|  | Rx | Pair 2 | Pins 3 & 6 |

## RS232 DIGITAL

| Equip. & Line | Pin 1 | RI |
| --- | --- | --- |
|  | Pin 2 | RTS |
|  | Pin 3 | RXD |
|  | Pin 4 | DCD |
|  | Pin 5 | TXD |
|  | Pin 6 | DTR |
|  | Pin 7 | Sig. Gnd. |
|  | Pin 8 | CTS |

LED Indicators:		TXD

RXD

Power

Power requirement:	24 Vdc nominal (20 - 30 Vdc) polarity insensitive, supplied via two-pin pluggable

screw terminal at rear of sub-rack

# FRONT & BACK PANEL DIAGRAM

## BLOCK DIAGRAM

## 4W ANALOG

## RS232 DIGITAL

## SPECIFICATIONS

## ANALOG

| Input/Output Impedance | 600  |
| --- | --- |
| Frequency Response |  0.2 dB, 300 Hz  4 kHz |
| Insertion Loss | 1 dB |
| Distortion | 0.3%, -20  0 dBm output level |
| Peak Voltage | 5.6Vp-p |

## RS232 DIGITAL

| Input Voltage | 20  30 Vdc, Polarity Insensitive, -ve Earth |
| --- | --- |
| Supply Current |  10 mA |
| Signal Delay |  3 s |
| Rise Time |  1.5 s |
| Data Rate |  115,200 bps |
| Nominal RS232 Signal Level Minimum RS232 Signal Level |  8V  5V |

## PART NUMBERS

RA-4865		AD-4865 1RU 19” Sub-rack

PC-4866		RS232 Isolation Card

PC-4796		4W Line Isolation Card

Designed and Manufactured in Australia

Design Two Thousand Pty Ltd

Melbourne Australia

Telephone: 03 9758 5933

Facsimile: 03 9758 5560

Email: info@design2000.com.au

Web site: www.design2000.com.au

### N468