# TELECOMMUTING HOME INTERFACE MODULE  HI-4690

## Description

The Telecommuting Home Interface Module (HIM) is used in conjunction with the NEAX Extender for Telecommuting purposes.

Without a HIM, the Telecommuter keeps the connection from the home office to the main NEAX PABX via the NEAX Extender by leaving the home phone ‘off-hook’. The Telecommuter signals the NEAX Extender to go off-hook, on-hook and hook flash on the PABX extension using DTMF commands.

By using an ‘on-hook’ dialling speaker telephone without a HIM, the Telecommuter can maintain the connection to the main NEAX PABX but will need to remember the dial commands that signal the NEAX Extender to answer, hang up and hook flash, and also remember not to actually hang up between calls if a continuous connection is required.

The Telecommuting Home Interface Module (HIM) solves these issues so that you can use the telephone like normal.

The HIM is situated at the Home Office and is connected in between the Telecommuter’s telephone socket and telephone. The HIM now does the work of sending answer, hang up and hook flash commands automatically. As a result, the Telecommuter’s job is made easier, where using an analogue telephone at the home office becomes exactly the same as in the main office.

When the Telecommuter is logged on for a working session and not on a call, the telephone can be placed on-hook. While the telephone is on hook, the HIM communicates with the NEAX Extender to hold the connection to the Home Office. When a call is presented to the telecommuter by the PABX or ACD, the NEAX Extender signals the HIM to ring. When the telephone is picked up, the HIM signals the NEAX Extender to loop the PABX extension. When the telephone is hung up, the HIM signals the NEAX Extender to hang up the PABX extension and keep the connection to the Extender in Hold Mode, or drop the connection in Dial Mode.

By utilising the HIM at the home office, the Telecommuter does not need to remember any special command codes and procedures, or to press special buttons to make and receive calls from the PABX.

## Statuses

Telephone		HIM			DTMF		NEAX Extender			PABX

(Line Two)				(Line One)

On-hook			Off hook		A A			Off-hook				On-hook

Ringing			Off hook		C			Off-hook				Ringing

Answer			Off hook		B			Off-hook				Off-hook

Hang up			Off hook		A A			Off-hook				On-hook

Off-hook		Off hook		B			Off-hook				Off-hook

Hook-flash		Hook flash		C C			Off-hook 				Hook-flash

# * #			On hook			D			On-hook					On-hook

## Operating Procedure

When first connecting the HIM, it will take about 30 seconds for the unit to ‘charge up’.

## To Establish the Telecommuting Session

- Dial the Extender and wait for answer.

- Enter your Operator number and PIN.

- Obtain dial tone and enter your ACD Log On code (if necessary).

- Hang up.

## To End the Telecommuting Session

- Lift handset, wait for dial tone and dial your ACD Log Off code (if necessary).

- Press # * #. Listen for “This telecommuting session is now over, goodbye”.

- Hang up.

OFFICE EXTENDER - TELECOMMUTING HOME INTERFACE MODULE  HI-4690

Description

The Telecommuting Home Interface Module (HIM) is used in conjunction with the Office Extender for Telecommuting purposes. It provides tone signalling over a non-dc transparent dial up telephone line.

Without a HIM, the Telecommuter controls the connection from the home office to the main office PABX via the Office Extender by making the Office Extender go off-hook, on-hook and hook flash on the PABX extension using DTMF commands.

Without a HIM, the Telecommuter will need to remember the dial commands that signal the Office Extender to answer, hook flash and hang up.

The Telecommuting Home Interface Module (HIM) solves these issues so that you can use the telephone like normal.

The HIM is situated at the Home Office and is connected in between the Telecommuter’s telephone socket and telephone. The HIM now does the work of sending answer, hook flash and hang up commands automatically. As a result, the Telecommuter’s job is made easier, where using an analogue telephone at the home office becomes exactly the same as in the main office.

When the Telecommuter is logged on for a working session and not on a call, the telephone can be placed on-hook. When a call is presented to the telecommuter by the PABX or ACD, the Office Extender signals the HIM to ring. When the telephone is picked up, the HIM signals the Office Extender to seize the PABX extension. When the telephone is hung up, the HIM signals the Office Extender to hang up the PABX extension.

By utilising the HIM at the home office, the Telecommuter does not need to remember any special command codes and procedures, or to press special buttons to make and receive calls from the PABX.

Telephone		HIM			DTMF		Office Extender			PABX

(Line Two)				(Line One)

On-hook			Off hook		A A			Off-hook				On-hook

Ringing			Off hook		C			Off-hook				Ringing

Answer			Off hook		B			Off-hook				Off-hook

Hang up			Off hook		A A			Off-hook				On-hook

Off-hook		Off hook		B			Off-hook				Off-hook

Hook-flash		Hook flash		C C			Off-hook 				Hook-flash

# * #			On hook			D			On-hook					On-hook

Operating Procedure

When first connecting the HIM, it will take about 30 seconds for the unit to initialize. During this period, any connected equipment must be ‘on-hook’.

To Establish the Telecommuting Session

- Dial the Extender and wait for answer.

- Enter your Operator number and PIN.

- Obtain dial tone and enter your ACD Log On code (if necessary).

- Hang up.

To End the Telecommuting Session

- Lift handset, wait for dial tone and dial your ACD Log Off code (if necessary).

- Press # * #. Listen for “This telecommuting session is now over, goodbye”. Repeat if necessary.

- Hang up.