#### LINE ISOLATOR / ODX DRIVER  TYPE LI-4302

#### DATA SHEET

##### GENERAL

The LI-4302 Line Isolation Unit is primarily a two wire dial through unit designed for use in conjunction with Customer Premises Equipment which is not capable of meeting the ACA requirements for line isolation and electrical safety.

The LI -4302 provides the appropriate isolation standards and allows connection to the PSTN of otherwise non permitted equipment (subject to the approval of the ACA for the combined LI-4302 and terminal equipment).

The LI-4302 can also be used as an Outdoor Extension (ODX) Driver for PABX and Key Telephone Systems. The LI-4302 regenerates the ringing signal and line feed to the remote equipment to meet the specifications below.

##### FEATURES

- Provides the necessary 3.5 kV ac isolation for more than one minute.

- Provides 48 V line feed to the equipment.

- Accepts ring voltage on the line side and regenerates ring voltage to the equipment

- Detects off hook condition and loops the line.

- Accepts decadic dialing from the equipment and transmits decadic dialing to line.

- Is capable of transmitting signals across the voice frequency spectrum in both directions including DTMF signals.

##### TELEPHONE SYSTEM TO PA INTERFACING

The LI-4302 with ECN 766 is suitable for interfacing a telephone or spare trunk (CO) port of a telephone system to a Public Address (PA) system for paging purposes. In this mode, the ‘equipment’ side is connected to the telephone system and the ‘line’ side is connected to the PA with line level input. Simply picking up the telephone or the designated line button will immediately connect you to the PA.

##### SPECIFICATIONS

| Isolation | >3.5 kV ac, >5 kV dc |
| --- | --- |
| Insertion loss | < 2 dB |
| Return loss (Terminated at customer end with AUSTEL Reference Impedance) | > 20 dB at 300 Hz > 30 dB from 600 Hz to 3400 Hz |
| Seizure, hold and pulse dialing current | Complies with AUSTEL Specification TS 002 |
| Pulse dialing response | < 150 micro seconds |
| Ring detect threshold | 30 V rms. @ 16 2/3 Hz |
| Ring output | 60 V ac rms. @ 50 Hz superimposed on 48 V dc, at nominal mains (minimum 20 V zero crossing) |
| Ring transfer response | < 50 milliseconds |
| Line seizure & decadic dialing detect current |  12 mA (pick up detected at 2.5k across the line but not at 10k DC loop resistance). |
| Output Equivalent Circuit | 48 V in series with 900 , current limited to 3540 mA, at nominal mains |
| Mains input | 240 Vac 50/60 Hz. (other voltages to order) |
| Power Consumption | 20 mA  5 VA maximum. |
| Mains cord length | 1.7 meters |
| AUSTEL Certified Component Listing No. | A95/TI/0215 |
| ACA Supplier’s Code | N 468 |
| Dimensions (single port) | W 110 mm x H 52 mm x D 85 mm |
| Ordering Code | LI-4302 (SY-4781) |

BLOCK DIAGRAM

MADE IN AUSTRALIA

DESIGN TWO THOUSAND PTY LTD

| TELEPHONE: 03 9758 5933 | FACSIMILE: 03 9758 5560 |
| --- | --- |
| E-MAIL: gen@design2000.com.au | WEBSITE: www.design2000.com.au |

##### N468