DMS CustomNet Spectrum Remote Monitor Extender

The RME allows authorised Test Officers at any location to remotely monitor telephone calls on any line.

Operating Procedure

- Dial Line 2 of the RME.

- Listen for “Welcome to CustomNet, please enter the password now”.

- Begin entering the four digit password within ten seconds. (You have only three attempts to enter the correct password).

- Listen for “Enter the full national number to be posted”.

- Enter the required number including the STD area code. If you make a mistake, press ** to start again. (Do not pause longer than eight seconds between digits, otherwise you will be disconnected).

- (The RME outputs (on the RS232 diagnostic port) the command string to bring up the line for monitoring MAPCI;MTC;LNS;LTP;LTPLTA;POST D <nnnnnnnnnn>;MONLTA

- This allows the number to be posted and the RME seizes Line 1 connected to the headset position).

- Monitor the line for activity for as long as you like.

- If required, press ** to return to the “Enter the full national number to be posted” prompt.

- To end the monitoring session, press #*# (or just hang up).

- (The RME outputs (on the RS232 diagnostic port) the command string QUIT ALL to release the connection for the next monitoring session and releases Line 1 connected to the headset position).

- Listen for “This session is now over, goodbye”

- Hang up.

Notes:

- To program the password from the RME keypad, press *#pppp#, where pppp is the four to eight digit password.

- The RME disconnects only on receipt of ‘Cut-off on Disconnect’ (COD) signal, DTMF #*# on Line 2 or manual keypad command <1>.

- The RME ignores DTMF and dc on Line 1 (Headset position).

- *67709600# = Master Reset Code

ACA Supplier’s Code N468