________________________________________________________________________________________________

#### TACT

## TA-7T

TRUNKED TWO-WAY RADIO / TELEPHONE INTERCONNECT

DESK TOP VERSION

________________________________________________________________________________________________

USER HANDBOOK

Issue 8,  MAY, 1998

ACA SUPPLIER’S CODE  N468

NEW ZEALAND TELEPERMIT PTC 210/96/005

________________________________________________________________________________________________

DESIGNED AND MANUFACTURED IN AUSTRALIA

### DESIGN TWO THOUSAND PTY LTD

________________________________________________________________________________________________

DESIGN TWO THOUSAND IS CERTIFIED TO AS3901/ISO9001/NZS9001

MADE IN AUSTRALIA

_______________________________________________________________________________________________

________________________________________________________________________________________________

TACT TA-7 T

MPT1327 TRUNKED MOBILE RADIO/TELEPHONE INTERCONNECT

USER HANDBOOK

| CONTENTS | ### PAGE |
| --- | --- |
| ##### Introduction | 1 |
| Standard Features | 1 |
| ##### Installation |  |
| Connecting TACT to the Telephone Line & to the Base Radio | 2 |
| ##### User Instructions |  |
| Calling a Radio from a Telephone | 2 |
| Calling a Telephone from a Radio | 2 |
| ##### Programming Options |  |
| Speed Dialing | 3 |
| Exchange line Access from a PABX | 3 |
| Type of Dialing | 3 |
| I.D. Announcement | 4 |
| Call Barring | 4 |
| Automatic Dialing of First Speed Call Number | 4 |
| Number of Radio Identity Digits | 4 |
| Automatic Diversion to a Radio | 5 |
| Call Clear Down and Follow-on Calls | 5 |
| Immediate Disconnect | 5 |
| Status Call for Calling Line Identification (CLI) | 6 |
| Voice Prompts | 6 |
| Alternate Language Voice Prompts | 6 |
| Programming Summary | 7 |
| Customer Programming Record | 8 |
| Appendix 1 - Network Call Set Up Mode | 9 |
| Appendix 2 - Call Logging Facility | 10 |
| Appendix 3 - Group Call Handling | 11 |
| Appendix 4 - ‘Base’ Radio Speaker Muting | 12 |
| Appendix 5 - Call Establishment Using a PC | 13 |

DESIGNED AND MADE IN AUSTRALIA

DESIGN TWO THOUSAND PTY LTD

________________________________________________________________________________________________

TACT TA-7T

INTRODUCTION

TACT, the Telephone Access Control Terminal, interfaces an MPT 1327 Trunking RF Control Base (Fixed Mobile) to any PABX/Key System internal analogue extension, or direct PSTN line.

Any mobile on the trunked system can call any telephone, using a DTMF microphone.

Similarly, any DTMF dialling telephone can selectively call any mobile.

Dual Tone Multi-Frequency (DTMF) is used to initiate calls both to and from mobiles.

STANDARD FEATURES

- Voice prompts and responses make the system easy and straight forward to use.

- DTMF/Decadic (Tone/Pulse) dialling.

- Call barring - programmable to enable/disable outside and long distance calls.

- Speed dialing of up to nine (or ninety nine) user pre-programmed telephone numbers.

- Last number re-dial.

- Automatic dialling of first Speed Call number if mobile does not have a DTMF microphone.

- Audio delays in both directions so that the beginning of sentences are not missed.

- Local Keypad programming.

- Full remote control. All service tone detection parameters, timers, and VOX timers and sensitivity are remotely programmable.

- Serial Interface.

- Standard interconnecting cable between RF control base radio and TACT (supplied).

- Calling party control of call connection/disconnection.

- Visual indicators include POWER, Tx (PTT), Rx, CONNECT (Line Looped), ON AIR, READY LEDs and a seven segment display.

- Call logging facility (optional).

- Bilingual voice prompts (optional).

- MAP 27 Interface (to order).

________________________________________________________________________________________________

INSTALLATION

CONNECTING TACT TO THE TELEPHONE LINE & TO THE BASE RADIO

TACT is connected to the PABX internal analogue extension or direct PSTN line using the RJ 12 to 605 line cord provided. The RJ 12 end is plugged into the socket at the back of TACT marked ‘LINE'. The 605 end (other plugs to order) is plugged into a working telephone socket.

When connecting TACT to a PABX internal analogue extension, this extension can be programmed to ‘ring' when a designated exchange line number is called.  This provides field access for callers calling in from any internal extension and/or from any outside line.

TACT is connected to the Base Radio using the Interconnecting cables supplied. (In the case of the GME TX 5240R, the control head is not used).

If you are using a Simoco or TAIT radio, the base microphone, which must be a standard mic (not a controller mic), may be plugged into the ‘MIC’ socket at the rear of TACT. Calling mobiles from the base is then possible using the TACT keypad.

In the case of Simoco radios, the Base Radio must be programmed for ‘Partial Off Air Mode’ (POACSU), for use with a ‘Controller mic’, NO ‘DTMF Overdial’ and not to respond to ‘Group Call’.

If you are using a MOTOROLA or NOKIA radio, it must have the MAP 27 Interface.

TACT is powered by a 12 V dc 1A power adaptor, or from the radio power supply unit where allowable.

USER INSTRUCTIONS

CALLING A TELEPHONE FROM A RADIO

Mobiles not already fitted with a DTMF dialling microphone, will need to have one fitted in order to make selective telephone calls.

- Call the base radio and listen for "Enter telephone number followed by hash (#)".

- Dial  the desired telephone number followed by #. If the number entered is  longer than  six  digits, TACT automatically pauses after the first digit  before  dialling  the rest of the number. This allows for PSTN exchange line access.

- Listen for "The number is nn...".

- If incorrect, press star (*) and start again.

- Listen for the normal ring tones etc.

- When the telephone party answers, proceed with conversation .

- To end the call, press the ‘CALL’ button on the front of the radio or simply  replace the mic on the cradle.

CALLING A RADIO FROM A TELEPHONE

Telephones used to call mobiles will need to have DTMF dialling capability. Portable DTMF Encoders or ‘Pocket Tone diallers’ may be used if the telephone does not dial DTMF.

- Dial the internal extension number or the exchange line number of TACT.

- Listen for "Enter mobile number followed by hash".

- Dial the two or three digit mobile number you wish to call followed by hash.

- Listen for the Trunked System's call progress tones.

- When the mobile answers, proceed with conversation.

- To end the call, either:

i)   Simply hang up, or :

ii)  Press # # for immediate disconnect, before hanging up.

________________________________________________________________________________________________

PROGRAMMING OPTIONS

All user programmed data, system options and tone detection parameters can be verified or changed via the inbuilt Keypad. Remote programming is also possible from any DTMF Telephone. Simply dial up TACT and wait for answer. Then enter # 0709650 # and listen for “Model T A 7 T, please enter command”. You can now program as normal.

Data retained in memory (EEPROM) can also be interrogated and altered remotely at higher speed when TACT is accessed by the central control computer at Design Two Thousand Pty Ltd.

SPEED DIALING

Abbreviated (or speed) dialling of up to 9 of your most frequently called telephone numbers.

To Store Speed Dial Numbers

- From the TACT Keypad, press *.

- Enter the required number (up to twenty digits).

- Press *.

- Press x, where x is the required memory location (1-9). The number will be displayed for confirmation.

- Note the number down in the MEMORY DIAL INDEX (last page).

- To check the number in memory, press * * x.

To Dial Speed Dial Numbers

- Call the base radio and listen for "Enter telephone number followed by hash (#)".

- Press #.

- Enter required memory location (1-9). You will hear the number being dialled.

- Proceed with the call.

Last Number Redial

- Call the base radio and listen for "Enter number followed by hash (#)".

- Press # #. You will hear the number being dialled.

- Proceed with the call.

EXCHANGE LINE ACCESS FROM A PABX

If the telephone number entered by a mobile user is longer than six digits, TACT automatically inserts a three second dialling pause after the first digit. This allows time for the second (exchange) dial tone to be obtained before TACT dials the outside number. If three seconds is not long enough to reliably obtain exchange dial tone, the dialling pause for exchange line access may be increased as follows:

From the Keypad of TACT:

- Press * 75 n #, where n = dialling pause in seconds (1 to 9s)

- To check the dialling pause, press * 75 #.

TYPE OF DIALLING (DTMF/DEC)

There are two types of dialling modes commonly used. Either DTMF (‘Touch  Tone’) or DECADIC (Pulse). Most telephone systems accept DTMF dialling, however some older systems may only detect Pulse dialling. TACT’s mode of dialling can be set as follows:

From the Keypad of TACT:

- Press * 61 0 # if the system is required to dial DTMF (Tone) OR,

- Press * 61 1 # if the system is required to dial DECADIC (Pulse).

- To check the mode of dialling, press * 61 #.

________________________________________________________________________________________________

I.D. ANNOUNCEMENT

When a call made from a radio to a telephone is answered, TACT can send an I.D. Announcement, "This  is a two-way radio call" to the telephone party, informing them of the radio call.

From the Keypad of TACT:

- Press * 80 1 #, to turn the I.D. Announcement on.

- Press * 80 0 #, to turn the I.D. Announcement off. (Default).

- To check whether the I.D. announcement is on or off, press * 80 #.

CALL BARRING

A convenient way of restricting mobile users from dialling outside local or long distance numbers is to limit the number of digits that they are allowed to dial.

From the Keypad of TACT:

- Press * 55 nn #, where nn can be 00-20.

- To check the number of digits allowed, press * 55 #.

NUMBER OF RADIO IDENTITY DIGITS (TWO OR THREE DIGIT IDENTITY)

Most fleets have radios with either a two or three digit identity. When calling a radio from a telephone, you are asked by TACT to "Enter Radio Number followed by hash". Many telephone users however are not familiar with the term hash (#). TACT may be programmed to accept either a two, or three digit radio identity. Telephone users are then not required to enter hash (#) after the radio number.

From the Keypad of TACT:

- Press * 56 nn #, where nn can be 00-16.

- To check the number of radio digits, press * 56 #.

Notes:

- If it is required to make inter-fleet calls from the telephone, do not program the number of radio digits to less than seven.

- Telephone users will be barred from accessing the PSTN via the Trunked Network. (Not to be confused with radios making PSTN calls via TACT which is possible).

AUTOMATIC DIALLING OF FIRST SPEED CALL NUMBER

If mobiles are not fitted with DTMF dialling microphones, it is possible for TACT to dial the first Speed Call number (see Speed dialing).

The mobile user calls the base radio and receives the voice prompt to "Enter telephone number followed by hash". If no DTMF digit is received within the preset time (1-29 seconds), TACT will dial the first pre-programmed Speed Call number.

To Program the Delay before automatically dialling the First Speed Call Number

From the Keypad of TACT:

- Press * 31 n #, where n is the delay in seconds (default 0 seconds).

- To check the delay, press * 31 #.

Notes:

- If the delay is set to 0 seconds, TACT will NOT automatically dial the first Speed Call number.

- If the delay is set to 30 seconds, TACT will immediately dial the first Speed Call number, and the voice response "The number is..." will NOT be announced before TACT commences dialling.

________________________________________________________________________________________________

AUTOMATIC DIVERSION TO A MOBILE

This mode is used to automatically divert incoming telephone calls to a mobile radio. When an incoming telephone call is received, TACT dials the mobile number stored in memory location nine (see ‘Speed dialing’). Callers into TACT will hear normal ringing tone until the call is answered at the mobile.

To Activate Diversion Facility to a Mobile:

From the Keypad of TACT:

- Press * 35 1 #.

Whether telephone calls are directed to the mobile or to the telephone at the base can be controlled by the Interconnect On/Off function.

To Turn Interconnect On:

From the keypad of TACT:

- Press * 51 1 #.

To Turn Interconnect Off:

From the keypad of TACT:

- Press * 51 0 #.

To De-activate Diversion Facility to a Mobile:

From the Keypad of TACT:

- Press * 35 0 #.

CALL CLEAR DOWN AND FOLLOW-ON CALLS

Automatic Disconnect

The TACT TA-7T will automatically disconnect a call when busy tone, dial tone, number unobtainable tone, or line loop current loss is detected on the telephone line.

Manual Clear Down and Follow-on Calls

In addition to the automatic call disconnect, there is a manual radio ‘clear down’ command: * #. (This works only from the telephone end to clear down the radio).

This enables the telephone party to make a ‘Follow-on’ call, without having to redial TACT each time consecutive calls to mobiles need to be made.

When a mobile is called from a telephone, and the mobile is not answering, you may press * #  and then dial the same or another mobile. This may be continued until an answer is found, or when the telephone is hung up.

Similarly, if a mobile ‘rings out’, you will hear "Mobile unattended" at which time you can press * # and dial the same or another mobile.

If you are on a call to a mobile and wish to dial another mobile, simply press * # (before or within 4 seconds of the mobile hanging up otherwise the connection will be lost), and dial another mobile.

Forced Disconnect

Please note that if you wish to force both the radio and telephone line to immediately disconnect without a follow-on call, press # # on the telephone or radio.

________________________________________________________________________________________________

STATUS CALL

When calling an unattended mobile, it is possible for the telephone party to leave a status message. When the person returns to the mobile, the status message may be used to identify the calling party so that the call can be returned.

The status message may simply be the last two digits of the extension number, some other two digit code for Calling Line Identification (CLI), or a normal status message.

To Leave a Status Message:

Mobile is not answering or did not answer:

- Press * # (within four seconds). You will hear "Enter mobile number followed by hash".

- Press * 0 nn * mm... #, where nn = 1-9, 10-30 (status message) and mm… =  mobile number to receive status message.

No attempt is made to actually call mobile, just leave status message:

- Call the base radio and listen for "Enter mobile number followed by hash (#)".

- Press * 0 nn * mm... #, where nn = 1-9, 10-30 (status message) and mm...  =  mobile number.

VOICE PROMPTS

The TA-7T usually guides the user with voice prompts and responses such as "Enter telephone number followed by hash", and "The number is ... ". Experienced users of the system may find the voice prompts repetitive, slowing, or of no functional value. The voice prompts can therefore be turned off partially or altogether.

To Alter the Voice Prompts

From the keypad of TACT:

- Press * 81 0 #  for no voice prompts, just a double ‘pip’ answer tone, or,

- Press * 81 1 # for full voice prompts and responses (default), or

- Press * 81 2 # for voice prompts but no number confirmation before dialling the telephone number, or

- Press * 81 3 # for voice prompts to the phone user only, and a double .’pip’ answer tone to the radio user.

ALTERNATE LANGUAGE VOICE PROMPTS

The TA-7T may be supplied with bilingual voice prompts. ie. in any two languages. eg. English and Indonesian, English and French, Spanish and French,  etc.. This must be specially arranged with your supplier prior to or at the time of ordering. (This is a future option at the time of writing).

From the keypad of TACT:

To Turn the Alternate Language Voice Prompts On:

Press * 82 1 #.

To Turn Standard (normally English) Voice Prompts On (Default):

Press * 82 0 #.

________________________________________________________________________________________________

PROGRAMMING SUMMARY

CODE			FUNCTION								FROM

nn... #			Call Radio from Telephone				Telephone

nn... #			Call Telephone from Radio				Radio

* nn... * x		Store Speed Call Number					Keypad

* * x			Check Speed Call Number					Keypad

# x				Dial Speed Call Number					Radio

# #				Last Number Redial						Radio

* #				Follow-on Call							Telephone

# #				Immediate disconnect						Telephone or radio

* 0 nn * mm... #	Status Call								Telephone

* 51 0 #			Interconnect Off							Keypad

* 51 1 #			Interconnect On							Keypad

* 61 0 #			DTMF (Tone) Dialling					Keypad

* 61 1 #			DECADIC (Pulse) Dialling				Keypad

* 61 #			Check Dialling Mode						Keypad

* 55 nn #		No. of Digits allowed for Call Barring		Keypad

* 55 #			Check number of Digits Allowed			Keypad

* 56 nn #		Number of Radio Identity Digits			Keypad

* 56 #			Check number of Radio Identity Digits		Keypad

* 31 n #			Program Delay before automatically

dialling first Speed Call number			Keypad

* 31 #			Check Delay								Keypad

* 32 0 #			Nine Speed Calls							Keypad

* 32 1 #			Ninety Nine Speed Calls					Keypad

* 35 0 #			Turn off Diversion to a Mobile				Keypad

* 35 1 #			Turn on Diversion to a Mobile				Keypad

* 36 0 #			Random Busy Tone Detect Off				Keypad

* 36 1 #			Random Busy Tone Detect On				Keypad

* 37 0 #			Auto Tone Detect Off						Keypad

* 37 1 #			Auto Tone Detect On						Keypad

* 75 n #			Dialling pause for exch. Access			Keypad

* 75 #			Check dialling pause (seconds)				Keypad

* 76 0 #			Calling Party Control off					Keypad

* 76 1 #			Calling Party Control on					Keypad

* 80 0 #			I.D. Announcement off					Keypad

* 80 1 #			I.D. Announcement on					Keypad

* 81 0 #			Voice prompts off						Keypad

* 81 1 #			Voice prompts on							Keypad

* 81 2 #			Voice prompts on but no number confirm	Keypad

* 81 3 #			Voice prompts to phone user only			Keypad

* 82 0 #			Standard (English) voice prompts			Keypad

* 82 1 #			Alternate language voice prompts			Keypad

* 90 nn #		Call Time limit							Keypad

* 48 #			EPROM Version Number					Keypad

# 0709650 #		Access Remote Programming mode		Telephone

# 00709650 #	Access Remote Programming mode		Radio

* 67 7096 00 #	Master Reset - Simoco					Keypad (CAUTION)

* 67 7096 01 #	Master Reset - GME						Keypad (CAUTION)

* 67 7096 02 #	Master Reset - TAIT						Keypad (CAUTION)

* 67 7096 03 #	Master Reset - Motorola MCX 1200		Keypad (CAUTION)

* 67 7096 04 #	Master Reset - Simoco Voice/Data			Keypad (CAUTION)

* 67 7096 05 #	Master Reset - Motorola SPECTRA		Keypad (CAUTION)

* 67 7096 06 #	Master Reset - NOKIA R40				Keypad (CAUTION)

Note:	Master Resets erase all customer programmed data and reset the TACT unit to the factory default condition.

________________________________________________________________________________________________

# TACT PROGRAMMING RECORD – TAIT T2040

| CUSTOMER |  | DATE |
| --- | --- | --- |
| CONTACT | PH/EXT No. | RADIO ID |

| COMMAND | ENTRY RANGE | FUNCTION | PAGE | SETTING |
| --- | --- | --- | --- | --- |

| SPEED DIALING MEMORIES (n…n up to 20 digits per memory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| *n…n* 1 | Max. 20 digits | Memory 1, automatically dialled after n seconds of no DTMF from mobile if * 31 n # | 3 |  |
| *n…n* 2 | Max. 20 digits | Memory 2 | 3 |  |
| *n…n* 3 | Max. 20 digits | Memory 3 | 3 |  |
| *n…n* 4 | Max. 20 digits | Memory 4 | 3 |  |
| *n…n* 5 | Max. 20 digits | Memory 5 | 3 |  |
| *n…n* 6 | Max. 20 digits | Memory 6 | 3 |  |
| *n…n* 7 | Max. 20 digits | Memory 7 | 3 |  |
| *n…n* 8 | Max. 20 digits | Memory 8 | 3 |  |
| *n…n* 9 | Max. 20 digits | Memory 9, phone calls are automatically diverted to Memory 9 (mobile number) if *35 1 # | 3 |  |

| SYSTEM FACILITIES |  |  |  |  |
| --- | --- | --- | --- | --- |
| * 31 n # | n = 0 n = 1 to 9 | Manual dialling of memory 1. n seconds before automatically dialling memory 1. | 4 |  |
| * 35 n # | n = 0 n = 1 | Manual calling of a mobile. Automatic diversion to a mobile number in memory 9. | 5 |  |
| * 55 n # | 00 to 20 | Number of digits that a mobile can dial. | 4 |  |
| * 56 n # | 00 to 16 | Number of Radio Identity Digits. | 4 |  |
| * 61 n # | n = 0 n = 1 | DTMF dialling. Decadic dialling. | 3 |  |
| * 76 n # | n = 0 n = 1 | Calling Party Control off. Calling Party Control on. |  |  |
| * 80 n # | n = 0 n = 1 | Incoming Radio Call Announcement off. Incoming Radio Call Announcement on. | 4 |  |

| MAINTENANCE COMMANDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| * 48 # |  | Software Version Number | 7 |  |
| *05 24 nnn # |  | Audio level from radio to telephone. Default = 025 | Tech. Man. |  |
| *05 25 nnn # |  | Audio level from telephone to radio. Default = 008 | Tech. Man. |  |
| * 6770960n # MASTER RESET |  | CAUTION. All factory default settings restored. See page 7 of Operator Manual. |  |  |

________________________________________________________________________________________________

APPENDIX 1 - NETWORK CALL SET UP MODE

The way in which calls are set up in an MPT 1327 Trunked Radio Network can vary depending on the particular network. This is referred to as the "Off Air Call Set Up (OACSU)" mode. It may be in the form of Full Off Air Call Set Up (FOACSU) or Partial  Off Air Call Set Up (POACSU).

If the Network is set for FOACSU (also referred to as manual answer), incoming calls are manually answered by TACT and then connected.

If the Network is set for POACSU (also referred to as automatic answer), incoming calls are automatically connected.

All MAP 27 TACT units will need to be programmed for either FOACSU or POACSU.

From the keypad of TACT:

To Select FOACSU (Default):

Press * 83 0 #. (Network requires incoming call answer for call to be connected)

To Select POACSU:

Press * 83 1 #. (Network connects incoming call immediately)

________________________________________________________________________________________________

APPENDIX 2 - CALL LOGGING FACILITY

All MAP 27 TACT units have a factory fitted option of providing caller or called party radio identification, global position (if fitted in vehicle), set up direction (phone to radio, radio to phone), telephone number dialled, call end and call duration.

This information is provided on a second RS 232 port - an optionally fitted DB 9 connector on the back panel of TACT marked "CALL DATA". Call logging information  (in ASCII) is sent to a Personal Computer where it can be directly displayed and/or processed as required.

TACT is capable of generating several Log numbering formats for call identification, the format of which must be selected to suit.

From the keypad of TACT:

To Select MPT 1327 Log Numbering Format (Default):

Press * 63 0 #.

To Select MPT 1343 Log Numbering Format:

Press * 63 1 #.

To Select Short form Log Numbering Format:

Press * 63 2 #.

To Select No Numbering Information:

Press * 63 3 #.

________________________________________________________________________________________________

APPENDIX 3 - GROUP CALL HANDLING

All MAP 27 TACT units may be connected to a ‘fixed’ mobile that is programmed to respond to group calls. There are several ways that TACT can handle a group call.

1.	Group Call ‘Ignore’

TACT denies telephone access to a group call. Any person dialling the TACT during a group call will receive a message "Call in progress, try again shortly".

2.	Group Call ‘Automatic Speed Call’

TACT detects a group call and immediately dials one of the pre-programmed speed call numbers so that a telephone party is automatically included in every group call.

3.	Group Call Manual ‘Join In’

Whenever a group call is in progress, TACT automatically connects any incoming telephone call. The telephone party is then able to take part in the group call.

Programming From the Keypad of TACT:

Group Call ‘Ignore’ (Default)

Press * 33 0 #.

Group Call ‘Automatic Speed Call’

Press * 33 n #, where n = 1-9, and 1-9 corresponds to the required speed call number.

Group Call Manual ‘Join In’

Press * 33 10 #.

________________________________________________________________________________________________

APPENDIX 4 - ‘BASE’ RADIO SPEAKER MUTING

TACT units which are connected to certain brands of MAP 27 fixed or base mobiles can mute the radio speaker when a call is connected to a telephone. At the end of the call the speaker reverts to volume level set by the volume control on the radio so that the radio can be used independently.

The speaker muting feature prevents the telephone party, when in ear shot of the ‘base’, from hearing ‘echo’ -unsynchronized (delayed) audio between the telephone and radio. It also means that other office staff are not disturbed by radio to telephone traffic, and also provides privacy.

Muting of the speaker is controlled by the selection of one of the following options:

1.	Speaker Mute for Group and Individual Radio/Telephone Calls (Default)

TACT is programmed to mute the base radio speaker whenever a group call or individual call is connected to the telephone. Note that if the ‘Group Call Handling’ is set to ‘Ignore’ (Appendix 3), no connection occurs and hence no muting.

2.	Speaker Mute for Group Radio/Telephone Calls

TACT is programmed to mute the base radio speaker only when a group call is connected to the telephone. Note that if the ‘Group Call Handling’ is set to ‘Ignore’ (Appendix 3), no connection occurs and hence no muting.

3.	Speaker Mute for Individual Radio/Telephone Calls

TACT is programmed to mute the base radio speaker only when an individual call is connected to the telephone.

4.	No Automatic Speaker Mute

The base radio speaker is always on, at a volume level set by the volume control on the radio.

Programming From the Keypad of TACT:

1.	Speaker Mute for Group and Individual Radio/Telephone Calls (Default)

* 84 0 #.

2.	Speaker Mute for Group Radio/Telephone Calls

* 84 1 #.

3.	Speaker Mute for Individual Radio/Telephone Calls

* 84 2 #.

4.	No Automatic Speaker Mute

* 84 3 #.

________________________________________________________________________________________________

APPENDIX 5 - CALL ESTABLISHMENT USING A PERSONAL COMPUTER

Description

If your TACT is fitted with the serial port used for the optional Call Logging Facility described in Appendix 2, then a PC may be used to establish calls requested by either a phone caller or a mobile radio caller.  In addition, the PC can unconditionally disconnect any established call.

The interconnect should be programmed to the Off state, with the command *51 0 # as shown in the Programming Summary. This allows the PC to take charge of call establishment rather than the radio or telephone user.

An operator located near the base radio answers radio or telephone calls, and places calls to the requested party, using a computer connected to TACT’s serial port. Typically the commands issued by the computer would be generated by suitable software accessing a data base of stored numbers, with provision for manually entering a mobile or telephone number. If required, this PC software is provided by you.

Connection

Your computer’s serial port can be connected with a simple three wire connection, Tx, Rx and Ground. The TACT’s standard serial port uses a 9 pin connector fitted as Data Communication Equipment (DCE). Direct connection is required if the PC’s serial port is a Data Terminating Equipment (DTE), which is the standard for IBM COM ports.  A null modem should be used if the PC uses a DCE connection, and a 9 pin to 25 pin adaptor if required.

The PC port must be set up for 9600 baud, 8 bits, no parity and one stop bit.

Commands

Three ASCII commands are available, which are not echoed back to the PC:

Call Mobile

The command is:		MOBILEnnnnnn#

Loop telephone line. Call mobile nnnnnn and pass control to TACT. At the end of the call TACT returns an idle state. Call logging described in Appendix 2 shows the set up and end of the call

Short form numbers and interfleet calls using MPT1343 format are currently supported.

Call Telephone

The command is:		PHONEnnnnnn#

Loop telephone and dial telephone number.  Pass control to TACT as before.

Unconditionally hangup

The command is:		HANGUP#

Unconditionally disconnect both phone and radio.

Possible Developments

Software Version 7033 and above of this facility provides for operator intervention for calls in both directions: radio-to-telephone and telephone-to-radio.  If required, future development could provide for a programmable option to select:

- Operator intervention in either direction or both.

- A programmed time during which calls could be intercepted by the operator, after which normal entering of the number would take place from telephone or radio.

If you require either of these enhancements, or any others, please discuss your requirements with Design Two Thousand Pty. Ltd.